df -h(disk file system)

tells the available and used space of the file system with where this is mounted.

/dev/root - is a device and it is mounted on /.

Created an extra volume and attached the volume with the corresponding instance.

After that ran df -h but still won’t able to see the created extra volume

Reason: Linux system doesn’t know where that volume is mounted.

To overcome the above issue, we have to run lsblk command that will list down all the blocks/volume.

lsblk

nvme01n1 is a volume that is already attached with the instance server

nvme1n1 is a volume that we bought an extra space.

But this extra volume/block is not mounted as per the above screenshot.

To mount the volume there is a directory called mnt(recommended)

Created a directory called data inside the mnt directory but unable to mount due to the file type.

Ran the below comman to make it file type compatible. - format

mkfs is make file system

mkfs -t ext4 /dev/nvme1n1

mount /dev/nvme1n1 /mnt/data

took another volume of 8 gb and mount it without login to the server:

If I do not want to login to the server, I can directly run the below command and it will the file system table.

vim /etc/fstab

automatically mount the file that will reach to the server directly and can be anything.

lvm - logical volume is a tool.

pvs: physical volumes

lvs: logical volumes

vgs: volume groups

Create a physical volume and volume group:

pvcreate /dev/nvnvme2n1

vgcreate test_group_volume nvme2n1

vgdisplay can will display all the information.

lvdisplay:

lsblk will show the logical volumes as below:

format the group:

Mount the volume:

Extend the volume:

lvextend -L +200M /dev/test_volume_group/tester_LV